TEACHING SHEET PEDRO – N°1
“Assembling the Pedro Robot”
(Duration: 2h – Difficulty: ★)

🎯 Learning Objective
Discover the Pedro robot, understand its basic operation, and explore its different control modes.

Students will learn:

- Basic concepts of robotics (programming, embedded systems, planetary gear systems)

- How a 3D printer works

- How to identify the components of a robot

- How to assemble a robot

🛠 Required Materials

- All Pedro robot parts printed in 3D

- 2 ball bearings

- 4 continuous rotation (360°) servomotors

- 1 micro USB cable

- 7.4V battery

- Pedro Rev3 Electronic Board

- PC (Windows, Linux, or OS X) with Arduino IDE installed

### Step 1 – Base

| - Place the battery in the base and close the cover - Place the electronic board in the middle of the base - Connect the battery to the board - Position the first servomotor in its slot (the cable should be oriented toward the board) | - Connect the servomotor to the board pins (Pin D5) - Place the first planetary gear system on top of the servo - Then place the gear in the center and press lightly so it attaches to the servo head - Turn on the board (On/Off button), the gear system should rotate in both directions |
| --- | --- |

### Step 2 – Shoulder

| - Insert the second servomotor into the robot’s shoulder - The servomotor should be oriented with the cable facing downward - Connect the shoulder support to the shoulder - Place the entire shoulder assembly on top of the planetary gear system | - Press until the shoulder is secured onto the planetary gear system - Gently rotate the shoulder left and right to check the planetary gear rotation - Screw the base with the 3 screws |
| --- | --- |